| Underbody Harness |  |  |  |  |
| --- | --- | --- | --- | --- |
| Ga. | Color | Harness |
| 18 | Blue w/yellow tracer | Cluster plug (A) TEMP ga to temp ga (ENG UNIT) terminal |
| 18 | White w/red tracer | Cluster plug (L) HI BEAM ind to foot dim switch (HI BEAM) term |
| 14 | Red w/white tracer | Foot dim switch (HI BEAM) term to jct block (HI BEAM) term |
| 18 | Green w/white tracer | Cluster plug (K) IGN to ign switch (IGN) term |
| 18 | Black w/white tracer | Cluster plug (J) RT TURN to dir signal switch term (RT TURN) |
| 16 | Black | Dir sig switch (RT TURN) to dir sig lamp (RT TURN) |
| 18 | Gray | Cluster plug (H) CHARGE IND to volt reg plug to alt (AUX) term |
| 18 | Purple | Cluster plug (G) OIL press ind to oil press sending unit (ENG) |
| 18 | Yellow w/black tracer | Cluster plug (E) LT TURN IND to dir sig switch (LT TURN) |
| 16 | Yellow | Dir sig switch (LT TURN) to dir sig lamp (LT TURN) |
| 18 | Black w/yellow tracer | Cluster plug (D) GROUND to instr panel ground (MTG) |
| 18 | White | Cluster plug (C) GAS GA to frame harness (GAS TANK GA) |
| 18 | Red w/blue tracer | Cluster plug (B) PANEL LTS to connector (1) to connector (2) to light switch term (I) |
| 12 | Red | IGN switch (B) term to solenoid (B) term |
| 14 | Dark blue | IGN switch (B) term to cigar lighter |
| 14 | Green | IGN switch (IGN) term to ballast |
| 14 | Green | Ballast to ign coil (POS) term |
| 16 | Light blue | IGN switch (START) term to solenoid (START) term |
| 14 | Red w/white tracer | Light switch batterry (B) term to solenoid (B) term |
| 14 | Green | Light switch (H) term to foot dim switch (B) term |
| 18 | Black w/yellow tracer | Light switch (D) term to dome light (neg) term |
| 16 | Brown | Stop light switch to light switch term (B-2) |
| 16 | Brown | Light switch term (B-2) to dome light (NEG) term |
| 16 | Yellow | Light switch (R) term to frame harness (TAIL LIGHT) |
| 16 | Yellow | Light switch (R) term to ign switch (LAMP) |
| 16 | Light blue | Light switch (P) term to jct block (PARK) term |
| 14 | Red w/white tracer | Heater switch (HI) term to heater (HI) term to blower motor |
| 16 | Light blue | Heater switch (LO) term to heater (LO) term |
| 16 | Black | Heater switch (MED term to heater (MED) term |
| 16 | Light Blue | Dir sig switch (LEFT REAR TURN) to frame harness (STOP LAMP LEFT) |
| 16 | Orange | Dir sig switch (RIGHT REAR TURN) to frame harness (STOP LAMP RIGHT) |
| 16 | Black | Foot dim switch (LO) to jct block (LO BEAM) |
| 12 | Red | Solenoid (B) term to alternator (OUTPUT) |
| 14 | Red w/white tracer | Alternator (OUTPUT) to horn (B) term |
| 18 | Black w/yellow tracer | Connector (HORN RELAY) to steer col connector (HORN BUTTON) |
| 16 | Brown | Stop light switch to dir sig switch (STOP LIGHT) term |
| 16 | Black w/white tracer | Volt regulator (G) term to alternator ground |
| 16 | Black w/white tracer | Volt regulator (F) term to laternator (F) term |
| Frame Harness |  |  |  |  |
| Ga. | Color | Harness |
| 16 | Orange | Connector dir sig switch to connector right rear (CROSS HARNESS) |
| 18 | White | Gas ga connecter (DASH UNIT) to connector gas ga (TANK UNIT) |
| 16 | Light blue | Connector dir sig switch to connector to left rear dir signal light |
| 16 | Yellow | Connector light switch to connector left tail light to connector right tail light |
| 16 | Yellow | Connector backup light switch to connector backup light left-right |
| Cross Harness |  |  |  |  |
| 16 | Yellow | Tail light connector(FRAME HARNESS) to right rear tail light and license light |
| 16 | Orange | Right rear dir sig connector (FRAME HARNESS) to right rear dir sig light |